Project 1: Snooping Cache Coherence Simulator

EECE 422 — Parallel Computer Architecture  |  Due: Monday, May 4, 2026, 11:59 PM

Groups of 2 (MSI protocol) or 3 (MESI protocol)

# 1. Overview

Cache coherence is a fundamental requirement of shared-memory multiprocessor systems. When multiple processor cores maintain private copies of a shared cache line, a coherence protocol must ensure that all copies remain consistent. Snooping protocols achieve this by broadcasting coherence transactions on a shared bus, allowing all caches to monitor and react to each other's requests.

In this project, you will implement a software simulator for a snooping cache coherence protocol in Python. Groups of two will implement the MSI (Modified-Shared-Invalid) protocol. Groups of three will implement the MESI (Modified-Exclusive-Shared-Invalid) protocol. You will use your simulator to run controlled experiments that measure coherence behavior under different workloads and system configurations.

# 2. Background

## 2.1 MSI Protocol (Groups of 2)

The MSI protocol assigns each cache line one of three states:

- Modified (M): The local cache holds the only valid copy and it has been written. The main memory copy is stale.

- Shared (S): One or more caches hold a valid read-only copy. Main memory is up to date.

- Invalid (I): The cache line is not present or has been invalidated.

Bus transactions include: BusRd (read request), BusRdX (read-exclusive, i.e., write request), and BusWB (writeback to memory). On a BusRdX, all other caches holding a Shared copy must transition to Invalid (invalidation).

## 2.2 MESI Protocol (Groups of 3)

MESI adds a fourth state to MSI:

- Exclusive (E): The local cache holds the only valid copy, identical to main memory. No other cache has this line. A write can proceed silently (without a BusRdX) by transitioning from E to M.

The Exclusive state reduces bus traffic by allowing a single-reader cache to upgrade to Modified without issuing a bus transaction. Groups of three must implement the full MESI state machine including silent upgrades and the associated bus signals (BusRd, BusRdX, BusUpgr, BusWB).

# 3. System Model

Your simulator must model the following system:

- N processor cores (N configurable, default N = 4), each with a private, write-invalidate cache.

- A single shared bus connecting all caches and main memory. Assume the bus is atomic: only one transaction occurs at a time, and all caches observe every transaction instantaneously.

- Direct-mapped caches with C lines each (C configurable, default C = 8). Each cache line holds one data word (the actual data value is irrelevant; model addresses only).

- Main memory as a flat address space of M addresses (M configurable, default M = 64).

- A workload trace as a sequence of memory access events of the form (core_id, operation, address), where operation is R (read) or W (write).

You do not need to model timing, pipelining, or bus arbitration. Process transactions one at a time in trace order. When a bus transaction is issued, all other caches snoop and respond before the next trace event is processed.

# 4. Implementation Requirements

## 4.1 Data Structures

At minimum, your simulator must maintain the following:

- A per-cache state table mapping each cache set index to a (tag, state) pair. State is one of {I, S, M} for MSI or {I, S, E, M} for MESI.

- A shared bus object that serializes transactions and notifies all caches.

- A main memory object that services cache misses and accepts writebacks.

- A statistics object that accumulates per-core and global counters (see Section 4.3).

## 4.2 Protocol State Machine

You must implement the full state machine for each protocol. The state machine must handle all processor-side events (PrRd, PrWr) and bus-side events (BusRd, BusRdX, BusUpgr for MESI). Specifically:

- On a PrRd miss: issue BusRd. All caches in state M must writeback and transition to S. Requesting cache transitions to S (MSI) or E/S (MESI, depending on whether any other cache responds).

- On a PrWr miss or upgrade from S: issue BusRdX (MSI/MESI) or BusUpgr (MESI, upgrade only). All caches holding S must invalidate. Requesting cache transitions to M.

- On a PrWr hit in state M: silent write, no bus transaction.

- On a PrWr hit in state E (MESI only): silent upgrade to M, no bus transaction.

- On eviction from state M: issue BusWB to write back to memory.

You are not required to model cache-to-cache transfers (intervention). On a BusRd where another cache holds M, that cache writebacks to memory and the requesting cache fetches from memory.

## 4.3 Statistics

Your simulator must collect the following per-experiment metrics:

- Read hits and read misses per core and globally.

- Write hits and write misses per core and globally.

- Number of invalidations issued (per core that receives an invalidation and per core that causes one).

- Total bus transactions (BusRd, BusRdX, BusWB; BusUpgr for MESI).

- (MESI only) Number of silent upgrades (E to M transitions).

## 4.4 Workload Traces

You must implement the following three canonical traces. Each trace consists of a sequence of (core_id, op, address) tuples. You may define the traces as Python lists in your code or load them from a file. Use the default configuration (N=4, C=8, M=64) unless otherwise noted in the experiment.

- Trace A — Read-Dominated Private Access: Each core repeatedly reads a disjoint set of addresses. No sharing between cores. Expected behavior: cold misses only, no invalidations, low bus traffic after warm-up.

- Trace B — Producer-Consumer: Core 0 writes a sequence of addresses; Cores 1–3 subsequently read the same addresses. Expected behavior: Core 0 transitions lines to M; subsequent reads from other cores trigger writebacks and sharing; significant bus traffic.

- Trace C — Migratory Sharing: A single address is written sequentially by each core in round-robin order (Core 0 writes, then Core 1 writes, then Core 2 writes, etc., repeated). Expected behavior: each write causes an invalidation of the previous owner; high invalidation and bus transaction counts.

You are free to add additional traces to support your experiments, but the three above are required.

# 5. Experiments

You must conduct the following experiments using your simulator. For each experiment, report the relevant statistics defined in Section 4.3. Present results in clearly labeled tables or figures. Each experiment must include a brief written analysis (see Section 6).

## Experiment 1: Baseline Protocol Behavior

Run all three traces (A, B, C) with the default configuration (N=4, C=8, M=64). Report all statistics from Section 4.3 in a single summary table. Verify that results match the expected behavior described in Section 4.4. Discuss any discrepancies.

## Experiment 2: Effect of Number of Cores

Using Trace B (Producer-Consumer), vary the number of cores N ∈ {2, 4, 6, 8}. Keep C=8 and M=64. Report total bus transactions and total invalidations as a function of N. Discuss how bus traffic scales with core count and what this implies for shared-bus snooping protocols.

## Experiment 3: Effect of Cache Size

Using Trace C (Migratory Sharing), vary the number of cache lines per cache C ∈ {4, 8, 16, 32}. Keep N=4 and M=64. Report miss rate and total bus transactions as a function of C. Discuss the relationship between cache capacity and coherence traffic.

## Experiment 4 (Groups of 3 — MESI only): Silent Upgrade Analysis

Using Trace A (Read-Dominated Private Access), compare your MESI simulator against the equivalent MSI behavior by disabling silent upgrades (i.e., treat all Exclusive lines as Shared). Report bus transactions under both configurations. Quantify the bus traffic reduction achieved by the Exclusive state.

# 6. Report Requirements

Your report must be submitted as a single PDF document. It must not exceed 10 pages, including all figures and tables. Use 11-point Times New Roman font, single-spaced, with 1-inch margins. If you cite any references, use IEEE citation style.

The report must contain the following four sections:

- Design: Describe your simulator architecture. Include a state transition diagram for your protocol (MSI or MESI). Explain your key data structure choices.

- Implementation: Describe non-obvious implementation decisions. Explain how you handle writebacks, invalidations, and (for MESI) silent upgrades. Do not include source code listings in the report.

- Experiments: Present all results from Section 5. Use tables or figures as appropriate. Label all axes, columns, and units clearly.

- Analysis: Interpret your results. Explain observed trends. Where results match theoretical expectations, state so explicitly. Where they do not, explain why.

# 7. Submission

Submit on Moodle by Monday, May 4, 2026, 11:59 PM. Your submission must include exactly two items:

- Report: A single PDF file named Project1_Report_[GroupMemberIDs].pdf.

- Code: A ZIP archive named Project1_Code_[GroupMemberIDs].zip containing all Python source files. Include a README.txt file with instructions for running the simulator and reproducing each experiment.

Only one member need make a submission with all group members listed in the report header.

# 8. Grading

| Component | Weight | Criteria |
| --- | --- | --- |
| Implementation | 40% | Correctness of protocol state machine; completeness of bus and cache logic; code clarity and structure. |
| Experiments | 40% | Design and execution of experiments; correctness of measurements; depth of analysis and interpretation. |
| Report | 20% | Clarity of writing; quality of figures and tables; adherence to format requirements. |

Note: Experiments will be graded on the quality of analysis, not merely on whether numbers are reported. A simulator that produces partially correct results but is accompanied by a thoughtful analysis of its limitations will be graded more favorably than one with correct numbers and no analysis.